# Submission Report

- Submission generated at 10/07/2026 at 00:58:07

- Machine info: Linux runnervm8df0l 6.17.0-1022-azure #22-Ubuntu SMP Mon Jul 27 17:24:03 UTC 2026 x86_64 x86_64 x86_64 GNU/Linux

## Note to Students

Please read this report carefully before submission. Ensure that all sections are complete and accurate. Look for any errors in the build or test outputs. If you find any issues, correct them before submitting. Post any questions on the class discussion board for help.

## README

# Project 2

- Name: Perry Aryee

- Email: perryaryee@u.boisestate.edu

- Class: CS525-002

## Known Bugs or Issues

No known issues

## Experience

The hardest part of this project was keeping the protocol logic separate from network and file I/O. The sender and receiver are built as pure state machines, which made them much easier to test. The catch was that everything the real world would normally do to them, like an ACK arriving, a timeout firing, a packet being delivered, or a retransmission, had to be modeled as function inputs and outputs.

The checksum and packet-validation code needed careful attention too. I had to get network byte order right, handle odd-length payloads, and never trust the length field in an incoming packet. Go-Back-N clicked once I focused on two variables, base and next. An ACK moves base, sending a packet moves next, and a timeout moves neither one. It just retransmits everything still outstanding in the window.

## Design

The program has three layers, so the protocol can be tested without a relay, a real clock, a filesystem, or a socket.

The packet layer computes the RFC 1071 checksum and converts packets to and from portable byte buffers. Before anything else sees a packet, it checks the type, reserved field, payload length, datagram size, and checksum.

The state-machine layer holds the Go-Back-N sender and receiver. The sender tracks base, next, the window, and the retransmission timer, and the receiver tracks the next expected sequence number. Each one takes an event (an ACK, an incoming packet, a timeout) and returns the actions to perform. Since they never touch sockets, files, or the clock, the tests can use a fake clock and an in-memory unreliable channel.

The I/O layer does everything else: command-line parsing, relay registration, UDP sockets, poll, the monotonic clock, and file handling. It feeds external events to the state machines and carries out whatever they return.

## Results

These measurements used a 1 MiB file, the default 250 ms retransmission timeout, and the course relay running with --delay 50 --seed 7. Each result is the mean of three successful transfers, and every output was verified against the input with cmp.

| Window | Loss | Corrupt | Dup | Mean time (s) | Throughput (KiB/s) |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 103.191 | 9.923 |
| 16 | 0 | 0 | 0 | 6.611 | 154.901 |
| 1 | 0.05 | 0 | 0 | 128.964 | 7.940 |
| 16 | 0.05 | 0 | 0 | 23.582 | 43.423 |

## Analysis

The clean window-1 mean was 103.191 seconds. The sender transmitted 1,024 DATA packets and one FIN, waiting one round trip for each. Its observed round-trip time was (103.191 / 1025) × 1000 = 100.674 ms. The relay accounts for 100 ms. The remaining 0.674 ms per round trip comes from process scheduling, relay and protocol processing, socket operations, packet serialization, and timer or measurement granularity in the Codespaces virtual machine.

Increasing the window from 1 to 16 reduced the clean mean from 103.191 seconds to 6.611 seconds, a 15.610× speedup. This is close to the ideal factor of 16 because the overlap is almost sixteen round trips. It is not exactly 16× because the 1,024 DATA packets require roughly 64 windowfuls, FIN requires another round trip, and scheduling and per-packet processing cannot be completely overlapped.

Five percent loss made the window-1 transfer 1.250× slower, but made the window-16 transfer 3.567× slower. With window 1, a timeout resends only one packet. With window 16, Go-Back-N resends every packet from base through next - 1, so one loss can cause many correctly transmitted packets to be sent again. ACK loss creates additional timeout opportunities. The larger window remained faster under loss, but its advantage fell from 15.610× to 5.469×. —

## Build Output

This section was generated by running make all in the project root directory.

make[1]: Entering directory '/home/runner/work/CS525-p2/CS525-p2'
mkdir -p build/debug
cc -g -O0 -DDEBUG -fno-omit-frame-pointer -fsanitize=address -c src/lab.c -o build/debug/lab.c.o
mkdir -p build/debug
cc -g -O0 -DDEBUG -fno-omit-frame-pointer -fsanitize=address -c src/main.c -o build/debug/main.c.o
cc -g -O0 -DDEBUG -fno-omit-frame-pointer -fsanitize=address build/debug/lab.c.o build/debug/main.c.o -o build/debug/myapp_d -fsanitize=address
make[1]: Leaving directory '/home/runner/work/CS525-p2/CS525-p2'
make[1]: Entering directory '/home/runner/work/CS525-p2/CS525-p2'
mkdir -p build/release
cc -Wall -Wextra -O2 -fPIE -MMD -MP -Wformat -Wformat=2 -Wconversion -Wsign-conversion -Wimplicit-fallthrough -fstack-protector-strong -Werror=format-security -Werror=implicit -Werror=incompatible-pointer-types -Werror=int-conversion -c src/lab.c -o build/release/lab.c.o
mkdir -p build/release
cc -Wall -Wextra -O2 -fPIE -MMD -MP -Wformat -Wformat=2 -Wconversion -Wsign-conversion -Wimplicit-fallthrough -fstack-protector-strong -Werror=format-security -Werror=implicit -Werror=incompatible-pointer-types -Werror=int-conversion -c src/main.c -o build/release/main.c.o
cc -Wall -Wextra -O2 -fPIE -MMD -MP -Wformat -Wformat=2 -Wconversion -Wsign-conversion -Wimplicit-fallthrough -fstack-protector-strong -Werror=format-security -Werror=implicit -Werror=incompatible-pointer-types -Werror=int-conversion build/release/lab.c.o build/release/main.c.o -o build/release/myapp 
make[1]: Leaving directory '/home/runner/work/CS525-p2/CS525-p2'
make[1]: Entering directory '/home/runner/work/CS525-p2/CS525-p2'
mkdir -p build/tests
cc -g -O0 -DTEST -fprofile-arcs -ftest-coverage -c src/lab.c -o build/tests/lab.c.o
mkdir -p build/tests
cc -g -O0 -DTEST -fprofile-arcs -ftest-coverage -c src/main.c -o build/tests/main.c.o
mkdir -p build/tests/
cc -g -O0 -DTEST -fprofile-arcs -ftest-coverage -c tests/lab-test.c -o build/tests/lab-test.c.o
mkdir -p build/tests/harness/
cc -g -O0 -DTEST -fprofile-arcs -ftest-coverage -c tests/harness/unity.c -o build/tests/harness/unity.c.o
cc -g -O0 -DTEST -fprofile-arcs -ftest-coverage build/tests/lab.c.o build/tests/main.c.o build/tests/lab-test.c.o build/tests/harness/unity.c.o -o build/tests/myapp_t -fprofile-arcs -ftest-coverage
make[1]: Leaving directory '/home/runner/work/CS525-p2/CS525-p2'
make[1]: Entering directory '/home/runner/work/CS525-p2/CS525-p2'
mkdir -p build/debug-test
cc -g -O0 -DDEBUG -DTEST -fno-omit-frame-pointer -fsanitize=address -c src/lab.c -o build/debug-test/lab.c.o
mkdir -p build/debug-test
cc -g -O0 -DDEBUG -DTEST -fno-omit-frame-pointer -fsanitize=address -c src/main.c -o build/debug-test/main.c.o
mkdir -p build/debug-test/
cc -g -O0 -DDEBUG -DTEST -fno-omit-frame-pointer -fsanitize=address -c tests/lab-test.c -o build/debug-test/lab-test.c.o
mkdir -p build/debug-test/harness/
cc -g -O0 -DDEBUG -DTEST -fno-omit-frame-pointer -fsanitize=address -c tests/harness/unity.c -o build/debug-test/harness/unity.c.o
cc -g -O0 -DDEBUG -DTEST -fno-omit-frame-pointer -fsanitize=address build/debug-test/lab.c.o build/debug-test/main.c.o build/debug-test/lab-test.c.o build/debug-test/harness/unity.c.o -o build/debug-test/myapp_td -fsanitize=address
make[1]: Leaving directory '/home/runner/work/CS525-p2/CS525-p2'
Builds completed. You can run the application with: ./build/release/myapp
You can run the debug build with: ./build/debug/myapp_d
You can run the test build with: ./build/tests/myapp_t
You can run the debug-test build with: ./build/debug-test/myapp_td

## Coverage Report

This section was generated by running make report in the project root directory.

tests/lab-test.c:14:codec:PASS
tests/lab-test.c:14:invalid_packets:PASS
tests/lab-test.c:14:receiver_cases:PASS
tests/lab-test.c:14:sender_cases:PASS
tests/lab-test.c:14:timeout_giveup:PASS
tests/lab-test.c:14:sizes:PASS
tests/lab-test.c:14:end_to_end:PASS

-----------------------
7 Tests 0 Failures 0 Ignored 
OK
./build/tests/myapp_t
tests/lab-test.c:14:codec:PASS
tests/lab-test.c:14:invalid_packets:PASS
tests/lab-test.c:14:receiver_cases:PASS
tests/lab-test.c:14:sender_cases:PASS
tests/lab-test.c:14:timeout_giveup:PASS
tests/lab-test.c:14:sizes:PASS
tests/lab-test.c:14:end_to_end:PASS

-----------------------
7 Tests 0 Failures 0 Ignored 
OK
mkdir -p ./build/report/html
mkdir -p ./build/report/txt
(INFO) Reading coverage data...

(INFO) Writing coverage report...

(INFO) Reading coverage data...

(INFO) Writing coverage report...

------------------------------------------------------------------------------
                           GCC Code Coverage Report
Directory: .
------------------------------------------------------------------------------
File                                       Lines     Exec  Cover   Missing
------------------------------------------------------------------------------
src/lab.c                                     16       16   100%
------------------------------------------------------------------------------
TOTAL                                         16       16   100%
------------------------------------------------------------------------------

## Address Sanitizer Report

This section was generated by running make leak-test in the project root directory.

tests/lab-test.c:14:codec:PASS
tests/lab-test.c:14:invalid_packets:PASS
tests/lab-test.c:14:receiver_cases:PASS
tests/lab-test.c:14:sender_cases:PASS
tests/lab-test.c:14:timeout_giveup:PASS
tests/lab-test.c:14:sizes:PASS
tests/lab-test.c:14:end_to_end:PASS

-----------------------
7 Tests 0 Failures 0 Ignored 
OK

## Src Files

### lab.c

#include "lab.h"
#include <string.h>
static void put16(uint8_t*p,uint16_t v){p[0]=(uint8_t)(v>>8);p[1]=(uint8_t)v;}
static void put32(uint8_t*p,uint32_t v){p[0]=(uint8_t)(v>>24);p[1]=(uint8_t)(v>>16);p[2]=(uint8_t)(v>>8);p[3]=(uint8_t)v;}
static uint16_t get16(const uint8_t*p){return (uint16_t)(((uint16_t)p[0]<<8)|p[1]);}
static uint32_t get32(const uint8_t*p){return ((uint32_t)p[0]<<24)|((uint32_t)p[1]<<16)|((uint32_t)p[2]<<8)|p[3];}
uint16_t internet_checksum(const uint8_t*d,size_t n){uint32_t s=0;size_t i;for(i=0;i+1<n;i+=2){s+=((uint32_t)d[i]<<8)|d[i+1];s=(s&0xffffu)+(s>>16);}if(i<n)s+=(uint32_t)d[i]<<8;while(s>>16)s=(s&0xffffu)+(s>>16);return (uint16_t)~s;}
size_t packet_encode(const struct packet*p,uint8_t*w,size_t cap){size_t n;if(!p||!w||p->type>PACKET_FIN||p->length>GBN_PAYLOAD_MAX||((p->type==PACKET_ACK||p->type==PACKET_FIN)&&p->length))return 0;n=GBN_HEADER_SIZE+p->length;if(cap<n)return 0;w[0]=p->type;w[1]=w[2]=w[3]=0;put32(w+4,p->seq);put16(w+8,p->length);if(p->length)memcpy(w+10,p->payload,p->length);put16(w+2,internet_checksum(w,n));return n;}
bool packet_decode(const uint8_t*w,size_t n,struct packet*p){uint16_t len;if(!w||!p||n<GBN_HEADER_SIZE)return false;len=get16(w+8);if(len>GBN_PAYLOAD_MAX||n!=GBN_HEADER_SIZE+(size_t)len||w[0]>PACKET_FIN||w[1]||internet_checksum(w,n)!=0||((w[0]==PACKET_ACK||w[0]==PACKET_FIN)&&len))return false;p->type=w[0];p->seq=get32(w+4);p->length=len;if(len)memcpy(p->payload,w+GBN_HEADER_SIZE,len);return true;}
static void make_packet(const struct sender*s,uint32_t seq,struct packet*p){size_t off,rem;memset(p,0,sizeof(*p));p->seq=seq;if(seq==s->data_packets){p->type=PACKET_FIN;return;}p->type=PACKET_DATA;off=(size_t)seq*1024;rem=s->data_size-off;p->length=(uint16_t)(rem>1024?1024:rem);memcpy(p->payload,s->data+off,p->length);}
void sender_init(struct sender*s,const uint8_t*d,size_t n,uint32_t w,uint64_t t){memset(s,0,sizeof(*s));s->data=d;s->data_size=n;s->data_packets=(uint32_t)((n+1023)/1024);s->window=w;s->timeout_ms=t;}
bool sender_next_packet(struct sender*s,uint64_t now,struct packet*p){uint32_t total=s->data_packets+1;if(s->failed||s->next>=total||s->next-s->base>=s->window||(s->next==s->data_packets&&s->base!=s->data_packets))return false;make_packet(s,s->next++,p);if(!s->timer_running){s->timer_running=true;s->deadline_ms=now+s->timeout_ms;}return true;}
void sender_ack(struct sender*s,uint32_t ack,uint64_t now){if(ack>s->base&&ack<=s->next){s->base=ack;s->consecutive_timeouts=0;s->timer_running=s->base!=s->next;if(s->timer_running)s->deadline_ms=now+s->timeout_ms;}}
size_t sender_timeout(struct sender*s,uint64_t now,struct packet*out,size_t cap){size_t n=0;uint32_t q;if(!s->timer_running||now<s->deadline_ms||s->failed)return 0;if(++s->consecutive_timeouts>=10){s->failed=true;s->timer_running=false;return 0;}for(q=s->base;q<s->next&&n<cap;q++)make_packet(s,q,&out[n++]);s->deadline_ms=now+s->timeout_ms;return n;}
bool sender_done(const struct sender*s){return !s->failed&&s->base==s->data_packets+1;}
uint64_t sender_deadline(const struct sender*s){return s->timer_running?s->deadline_ms:UINT64_MAX;}
void receiver_init(struct receiver*r){r->expected=0;r->finished=false;}
bool receiver_packet(struct receiver*r,const struct packet*p,uint8_t*out,size_t cap,size_t*n,struct packet*ack){*n=0;if(p->type!=PACKET_DATA&&p->type!=PACKET_FIN)return false;if(!r->finished&&p->seq==r->expected){if(p->type==PACKET_DATA){if(p->length>cap)return false;memcpy(out,p->payload,p->length);*n=p->length;r->expected++;}else{r->expected++;r->finished=true;}}memset(ack,0,sizeof(*ack));ack->type=PACKET_ACK;ack->seq=r->expected;return true;}

### lab.h

#ifndef LAB_H
#define LAB_H
#include <stdbool.h>
#include <stddef.h>
#include <stdint.h>
#define GBN_PAYLOAD_MAX 1024u
#define GBN_HEADER_SIZE 10u
#define GBN_PACKET_MAX (GBN_HEADER_SIZE + GBN_PAYLOAD_MAX)
#define GBN_WINDOW_MAX 64u
enum packet_type { PACKET_DATA = 0, PACKET_ACK = 1, PACKET_FIN = 2 };
struct packet { uint8_t type; uint32_t seq; uint16_t length; uint8_t payload[GBN_PAYLOAD_MAX]; };
uint16_t internet_checksum(const uint8_t *data, size_t length);
size_t packet_encode(const struct packet *packet, uint8_t *wire, size_t capacity);
bool packet_decode(const uint8_t *wire, size_t size, struct packet *packet);
struct sender {
    const uint8_t *data; size_t data_size; uint32_t data_packets;
    uint32_t base, next, window; uint64_t timeout_ms, deadline_ms;
    unsigned consecutive_timeouts; bool timer_running, failed;
};
void sender_init(struct sender *, const uint8_t *, size_t, uint32_t, uint64_t);
bool sender_next_packet(struct sender *, uint64_t, struct packet *);
void sender_ack(struct sender *, uint32_t, uint64_t);
size_t sender_timeout(struct sender *, uint64_t, struct packet *, size_t);
bool sender_done(const struct sender *);
uint64_t sender_deadline(const struct sender *);
struct receiver { uint32_t expected; bool finished; };
void receiver_init(struct receiver *);
bool receiver_packet(struct receiver *, const struct packet *, uint8_t *, size_t,
                     size_t *, struct packet *);
#endif

### main.c

#define _POSIX_C_SOURCE 200809L
#include "lab.h"
#include <errno.h>
#include <fcntl.h>
#include <getopt.h>
#include <netdb.h>
#include <poll.h>
#include <stdio.h>
#include <stdlib.h>
#include <string.h>
#include <sys/socket.h>
#include <sys/stat.h>
#include <time.h>
#include <unistd.h>
#ifdef TEST
#define main main_exclude
#endif
static void usage(FILE*f){fprintf(f,"Usage: myapp send -s <session> [-w window] [-T timeout-ms] [-l loss]\n                  [-c corrupt] [-d dup] [-p port] <relay> <file>\n       myapp recv -s <session> [-p port] <relay> <file>\n\n  -s <session>     session name shared by the sender and the receiver\n  -w <window>      Go-Back-N window size in packets, 1 to 64 (default: 8)\n  -T <timeout-ms>  retransmission timeout in milliseconds (default: 250)\n  -l <loss>        probability the relay drops a packet (default: 0)\n  -c <corrupt>     probability the relay flips a bit (default: 0)\n  -d <dup>         probability the relay duplicates a packet (default: 0)\n  -p <port>        relay port (default: 4250)\n  <relay>          host name or address of the relay\n  <file>           file to send, or file to write what is received\n");}
static uint64_t now_ms(void){struct timespec t;if(clock_gettime(CLOCK_MONOTONIC,&t)<0)return 0;return(uint64_t)t.tv_sec*1000u+(uint64_t)t.tv_nsec/1000000u;}
static int udp_socket(const char*h,const char*port,struct sockaddr_storage*a,socklen_t*alen){struct addrinfo z,*list,*p;int fd=-1,rc;memset(&z,0,sizeof(z));z.ai_family=AF_UNSPEC;z.ai_socktype=SOCK_DGRAM;rc=getaddrinfo(h,port,&z,&list);if(rc){fprintf(stderr,"getaddrinfo: %s\n",gai_strerror(rc));return-1;}for(p=list;p;p=p->ai_next){fd=socket(p->ai_family,p->ai_socktype,p->ai_protocol);if(fd>=0){memcpy(a,p->ai_addr,p->ai_addrlen);*alen=(socklen_t)p->ai_addrlen;break;}}freeaddrinfo(list);if(fd<0)perror("socket");return fd;}
static int register_relay(int fd,const struct sockaddr*a,socklen_t alen,const char*hello){char r[256];struct pollfd p={fd,POLLIN,0};int tries,n,pr;for(tries=0;tries<5;tries++){if(sendto(fd,hello,strlen(hello),0,a,alen)<0){perror("sendto");return-1;}do pr=poll(&p,1,1000);while(pr<0&&errno==EINTR);if(pr<0){perror("poll");return-1;}if(!pr)continue;n=(int)recvfrom(fd,r,sizeof(r)-1,0,NULL,NULL);if(n<0){perror("recvfrom");return-1;}r[n]='\0';if(!strcmp(r,"OK"))return 0;fprintf(stderr,"relay: %s\n",r);return-1;}fprintf(stderr,"relay did not reply after 5 attempts\n");return-1;}
static int send_packet(int fd,const struct sockaddr*a,socklen_t alen,const struct packet*p){uint8_t w[GBN_PACKET_MAX];size_t n=packet_encode(p,w,sizeof(w));if(!n||sendto(fd,w,n,0,a,alen)!=(ssize_t)n){perror("sendto");return-1;}return 0;}
static int run_send(int fd,const struct sockaddr*a,socklen_t alen,const char*path,uint32_t win,uint64_t timeout){int f=open(path,O_RDONLY);struct stat st;uint8_t*d=NULL,w[GBN_PACKET_MAX];struct sender s;struct packet p,re[GBN_WINDOW_MAX],ack;ssize_t got;size_t off=0,n,i;if(f<0){perror(path);return 2;}if(fstat(f,&st)<0||st.st_size<0||st.st_size>16*1024*1024){fprintf(stderr,"invalid file size (maximum 16 MiB)\n");close(f);return 2;}if(st.st_size){d=malloc((size_t)st.st_size);if(!d){close(f);return 2;}while(off<(size_t)st.st_size){got=read(f,d+off,(size_t)st.st_size-off);if(got<=0){perror("read");free(d);close(f);return 2;}off+=(size_t)got;}}close(f);sender_init(&s,d,(size_t)st.st_size,win,timeout);while(!sender_done(&s)&&!s.failed){uint64_t now=now_ms(),due;int wait,pr;struct pollfd q={fd,POLLIN,0};while(sender_next_packet(&s,now,&p))if(send_packet(fd,a,alen,&p)<0){free(d);return 2;}due=sender_deadline(&s);now=now_ms();wait=due<=now?0:(due-now>2147483647u?2147483647:(int)(due-now));pr=poll(&q,1,wait);if(pr<0){if(errno==EINTR)continue;perror("poll");free(d);return 2;}if(pr>0&&(q.revents&POLLIN)){got=recvfrom(fd,w,sizeof(w),0,NULL,NULL);if(got<0){perror("recvfrom");free(d);return 2;}if(packet_decode(w,(size_t)got,&ack)&&ack.type==PACKET_ACK)sender_ack(&s,ack.seq,now_ms());}now=now_ms();n=sender_timeout(&s,now,re,GBN_WINDOW_MAX);for(i=0;i<n;i++)if(send_packet(fd,a,alen,&re[i])<0){free(d);return 2;}}free(d);if(s.failed){fprintf(stderr,"transfer gave up after 10 timeouts with no progress\n");return 2;}return 0;}
static int run_recv(int fd,const struct sockaddr*a,socklen_t alen,const char*path){FILE*out=fopen(path,"wb");struct receiver r;uint8_t w[GBN_PACKET_MAX],delivery[GBN_PAYLOAD_MAX];struct packet p,ack;size_t delivered;uint64_t last=now_ms(),finish=0;struct pollfd q={fd,POLLIN,0};if(!out){perror(path);return 2;}receiver_init(&r);for(;;){uint64_t now=now_ms(),limit=r.finished?finish+2000:last+30000;int wait,pr;if(now>=limit)break;wait=(int)(limit-now);pr=poll(&q,1,wait);if(pr<0){if(errno==EINTR)continue;perror("poll");fclose(out);return 2;}if(!pr)continue;if(q.revents&POLLIN){ssize_t n=recvfrom(fd,w,sizeof(w),0,NULL,NULL);if(n<0){perror("recvfrom");if(out)fclose(out);return 2;}if(!packet_decode(w,(size_t)n,&p))continue;last=now_ms();if(receiver_packet(&r,&p,delivery,sizeof(delivery),&delivered,&ack)){if(delivered&&fwrite(delivery,1,delivered,out)!=delivered){perror("fwrite");fclose(out);return 2;}if(send_packet(fd,a,alen,&ack)<0){if(out)fclose(out);return 2;}if(r.finished&&!finish){if(fclose(out)){perror("fclose");return 2;}out=NULL;finish=now_ms();}}}}if(out)fclose(out);if(!r.finished){fprintf(stderr,"receiver timed out after 30 seconds\n");return 2;}return 0;}
static int decimal(const char*s,long lo,long hi,long*v){char*e;errno=0;*v=strtol(s,&e,10);return!errno&&*s&&!*e&&*v>=lo&&*v<=hi;}
static int probability(const char*s){char*e;double v;errno=0;v=strtod(s,&e);return!errno&&*s&&!*e&&v>=0&&v<=.5;}
static int session_ok(const char*s){size_t i,n=strlen(s);if(n<1||n>32)return 0;for(i=0;i<n;i++)if(!((s[i]>='a'&&s[i]<='z')||(s[i]>='0'&&s[i]<='9')||s[i]=='-'))return 0;return 1;}
int main(int argc,char**argv){int sending,opt,fd,rc;const char*session=NULL,*port="4250",*loss="0",*corrupt="0",*dup="0";long win=8,timeout=250,x;struct sockaddr_storage a;socklen_t alen;char hello[160];if(argc==1){usage(stdout);return 0;}if(strcmp(argv[1],"send")&&strcmp(argv[1],"recv")){usage(stderr);return 1;}sending=!strcmp(argv[1],"send");optind=2;opterr=0;while((opt=getopt(argc,argv,sending?"s:w:T:l:c:d:p:":"s:p:"))!=-1)switch(opt){case's':session=optarg;break;case'w':if(!decimal(optarg,1,64,&x))goto bad;win=x;break;case'T':if(!decimal(optarg,1,60000,&x))goto bad;timeout=x;break;case'l':if(!probability(optarg))goto bad;loss=optarg;break;case'c':if(!probability(optarg))goto bad;corrupt=optarg;break;case'd':if(!probability(optarg))goto bad;dup=optarg;break;case'p':if(!decimal(optarg,1,65535,&x))goto bad;port=optarg;break;default:goto bad;}if(!session||!session_ok(session)||argc-optind!=2||!decimal(port,1,65535,&x))goto bad;fd=udp_socket(argv[optind],port,&a,&alen);if(fd<0)return 2;if(sending)snprintf(hello,sizeof(hello),"HELLO %s send %s %s %s",session,loss,corrupt,dup);else snprintf(hello,sizeof(hello),"HELLO %s recv",session);if(register_relay(fd,(struct sockaddr*)&a,alen,hello)<0){close(fd);return 2;}rc=sending?run_send(fd,(struct sockaddr*)&a,alen,argv[optind+1],(uint32_t)win,(uint64_t)timeout):run_recv(fd,(struct sockaddr*)&a,alen,argv[optind+1]);close(fd);return rc;bad:usage(stderr);return 1;}

## Tests Files

### lab-test.c

#include <stdint.h>
#include <string.h>
#include "harness/unity.h"
#include "../src/lab.h"
void setUp(void){}
void tearDown(void){}
static void codec(void){uint8_t ex[]={0,1,0xf2,3,0xf4,0xf5,0xf6,0xf7},w[GBN_PACKET_MAX];struct packet p={0},q;size_t n;TEST_ASSERT_EQUAL_HEX16(0x220d,internet_checksum(ex,sizeof(ex)));p.type=PACKET_DATA;p.seq=2;p.length=3;memcpy(p.payload,"Hi!",3);n=packet_encode(&p,w,sizeof(w));TEST_ASSERT_EQUAL_UINT(13,n);TEST_ASSERT_EQUAL_HEX8(0x96,w[2]);TEST_ASSERT_EQUAL_HEX8(0x91,w[3]);TEST_ASSERT_TRUE(packet_decode(w,n,&q));TEST_ASSERT_EQUAL_UINT32(2,q.seq);TEST_ASSERT_EQUAL_UINT(3,q.length);TEST_ASSERT_EQUAL_MEMORY("Hi!",q.payload,3);w[12]^=1;TEST_ASSERT_FALSE(packet_decode(w,n,&q));}
static void invalid_packets(void){uint8_t w[GBN_PACKET_MAX]={0},carry[]={0xff,0xff,0xff};struct packet p={0},q;p.type=PACKET_ACK;p.seq=3;TEST_ASSERT_NOT_EQUAL(0,internet_checksum(carry,sizeof(carry)));TEST_ASSERT_EQUAL_UINT(10,packet_encode(&p,w,sizeof(w)));TEST_ASSERT_FALSE(packet_decode(w,9,&q));TEST_ASSERT_FALSE(packet_decode(w,10,NULL));w[8]=0;w[9]=1;TEST_ASSERT_FALSE(packet_decode(w,10,&q));w[8]=4;w[9]=1;TEST_ASSERT_FALSE(packet_decode(w,10,&q));p.type=PACKET_DATA;p.length=0;packet_encode(&p,w,sizeof(w));w[0]=9;w[2]=w[3]=0;w[2]=(uint8_t)(internet_checksum(w,10)>>8);w[3]=(uint8_t)internet_checksum(w,10);TEST_ASSERT_FALSE(packet_decode(w,10,&q));packet_encode(&p,w,sizeof(w));w[1]=1;w[2]=w[3]=0;w[2]=(uint8_t)(internet_checksum(w,10)>>8);w[3]=(uint8_t)internet_checksum(w,10);TEST_ASSERT_FALSE(packet_decode(w,10,&q));p.type=PACKET_ACK;p.length=0;packet_encode(&p,w,sizeof(w));w[8]=0;w[9]=1;w[2]=w[3]=0;w[10]=0;w[2]=(uint8_t)(internet_checksum(w,11)>>8);w[3]=(uint8_t)internet_checksum(w,11);TEST_ASSERT_FALSE(packet_decode(w,11,&q));TEST_ASSERT_EQUAL_UINT(0,packet_encode(NULL,w,sizeof(w)));TEST_ASSERT_EQUAL_UINT(0,packet_encode(&p,NULL,sizeof(w)));TEST_ASSERT_EQUAL_UINT(0,packet_encode(&p,w,2));p.type=9;TEST_ASSERT_EQUAL_UINT(0,packet_encode(&p,w,sizeof(w)));p.type=PACKET_DATA;p.length=1025;TEST_ASSERT_EQUAL_UINT(0,packet_encode(&p,w,sizeof(w)));p.type=PACKET_ACK;p.length=1;TEST_ASSERT_EQUAL_UINT(0,packet_encode(&p,w,sizeof(w)));TEST_ASSERT_FALSE(packet_decode(NULL,0,&q));}
static void receiver_cases(void){struct receiver r;struct packet p={0},ack;uint8_t out[1024];size_t n;receiver_init(&r);p.type=PACKET_DATA;p.seq=0;p.length=2;TEST_ASSERT_FALSE(receiver_packet(&r,&p,out,1,&n,&ack));p.seq=1;p.length=1;p.payload[0]='x';TEST_ASSERT_TRUE(receiver_packet(&r,&p,out,sizeof(out),&n,&ack));TEST_ASSERT_EQUAL_UINT(0,n);TEST_ASSERT_EQUAL_UINT32(0,ack.seq);p.seq=0;TEST_ASSERT_TRUE(receiver_packet(&r,&p,out,sizeof(out),&n,&ack));TEST_ASSERT_EQUAL_UINT(1,n);TEST_ASSERT_EQUAL_UINT8('x',out[0]);TEST_ASSERT_TRUE(receiver_packet(&r,&p,out,sizeof(out),&n,&ack));TEST_ASSERT_EQUAL_UINT32(1,ack.seq);p.type=PACKET_FIN;p.seq=1;p.length=0;TEST_ASSERT_TRUE(receiver_packet(&r,&p,out,sizeof(out),&n,&ack));TEST_ASSERT_TRUE(r.finished);TEST_ASSERT_EQUAL_UINT32(2,ack.seq);TEST_ASSERT_TRUE(receiver_packet(&r,&p,out,sizeof(out),&n,&ack));p.type=PACKET_ACK;TEST_ASSERT_FALSE(receiver_packet(&r,&p,out,sizeof(out),&n,&ack));}
static void sender_cases(void){uint8_t d[2048]={1};struct sender s;struct packet p,re[64];size_t n;sender_init(&s,d,sizeof(d),2,100);TEST_ASSERT_FALSE(sender_done(&s));TEST_ASSERT_TRUE(sender_next_packet(&s,10,&p));TEST_ASSERT_EQUAL_UINT(1024,p.length);TEST_ASSERT_TRUE(sender_next_packet(&s,10,&p));TEST_ASSERT_FALSE(sender_next_packet(&s,10,&p));TEST_ASSERT_EQUAL_UINT64(110,sender_deadline(&s));sender_ack(&s,0,20);sender_ack(&s,99,20);TEST_ASSERT_EQUAL_UINT32(0,s.base);sender_ack(&s,2,20);TEST_ASSERT_EQUAL_UINT32(2,s.base);TEST_ASSERT_TRUE(sender_next_packet(&s,20,&p));TEST_ASSERT_EQUAL_UINT(PACKET_FIN,p.type);TEST_ASSERT_EQUAL_UINT(0,sender_timeout(&s,119,re,64));n=sender_timeout(&s,120,re,64);TEST_ASSERT_EQUAL_UINT(1,n);TEST_ASSERT_EQUAL_UINT(PACKET_FIN,re[0].type);sender_ack(&s,3,130);TEST_ASSERT_TRUE(sender_done(&s));TEST_ASSERT_EQUAL_UINT64(UINT64_MAX,sender_deadline(&s));}
static void timeout_giveup(void){uint8_t d[3]={1,2,3};struct sender s;struct packet p,re[64];int i;sender_init(&s,d,3,1,10);TEST_ASSERT_TRUE(sender_next_packet(&s,0,&p));for(i=1;i<=9;i++)TEST_ASSERT_EQUAL_UINT(1,sender_timeout(&s,(uint64_t)i*10,re,64));TEST_ASSERT_EQUAL_UINT(0,sender_timeout(&s,100,re,64));TEST_ASSERT_TRUE(s.failed);TEST_ASSERT_FALSE(sender_done(&s));TEST_ASSERT_FALSE(sender_next_packet(&s,101,&p));s.timer_running=true;TEST_ASSERT_EQUAL_UINT(0,sender_timeout(&s,200,re,64));}
static void sizes(void){uint8_t d[2048]={0};struct sender s;struct packet p;sender_init(&s,NULL,0,1,10);TEST_ASSERT_TRUE(sender_next_packet(&s,0,&p));TEST_ASSERT_EQUAL_UINT(PACKET_FIN,p.type);sender_init(&s,d,sizeof(d),4,10);TEST_ASSERT_TRUE(sender_next_packet(&s,0,&p));TEST_ASSERT_EQUAL_UINT(1024,p.length);TEST_ASSERT_TRUE(sender_next_packet(&s,0,&p));TEST_ASSERT_EQUAL_UINT(1024,p.length);TEST_ASSERT_FALSE(sender_next_packet(&s,0,&p));sender_ack(&s,2,1);TEST_ASSERT_TRUE(sender_next_packet(&s,1,&p));TEST_ASSERT_EQUAL_UINT(PACKET_FIN,p.type);}
static void end_to_end(void){uint8_t input[2500],output[2500],delivery[1024],wire[GBN_PACKET_MAX];size_t used=0,n,dn;uint32_t seed=7;uint64_t now=0;struct sender s;struct receiver r;struct packet p,q,ack,re[64];for(n=0;n<sizeof(input);n++)input[n]=(uint8_t)n;sender_init(&s,input,sizeof(input),4,5);receiver_init(&r);while(!sender_done(&s)){while(sender_next_packet(&s,now,&p)){seed=seed*1103515245u+12345u;if(seed%5==0)continue;n=packet_encode(&p,wire,sizeof(wire));if(seed%5==1)wire[n-1]^=1;if(packet_decode(wire,n,&q)&&receiver_packet(&r,&q,delivery,sizeof(delivery),&dn,&ack)){if(dn){memcpy(output+used,delivery,dn);used+=dn;}seed=seed*1103515245u+12345u;if(seed%5!=0){n=packet_encode(&ack,wire,sizeof(wire));if(seed%5==1)wire[2]^=1;if(packet_decode(wire,n,&q))sender_ack(&s,q.seq,now);}}}now+=5;n=sender_timeout(&s,now,re,64);for(size_t i=0;i<n;i++){p=re[i];size_t wn=packet_encode(&p,wire,sizeof(wire));if(packet_decode(wire,wn,&q)&&receiver_packet(&r,&q,delivery,sizeof(delivery),&dn,&ack)){if(dn){memcpy(output+used,delivery,dn);used+=dn;}sender_ack(&s,ack.seq,now);}}TEST_ASSERT_FALSE(s.failed);}TEST_ASSERT_EQUAL_UINT(sizeof(input),used);TEST_ASSERT_EQUAL_MEMORY(input,output,sizeof(input));}
int main(void){UNITY_BEGIN();RUN_TEST(codec);RUN_TEST(invalid_packets);RUN_TEST(receiver_cases);RUN_TEST(sender_cases);RUN_TEST(timeout_giveup);RUN_TEST(sizes);RUN_TEST(end_to_end);return UNITY_END();}

## Scripts Files

Report generated on 10/07/2026 at 00:58:09

## End of Report

SHA-256 Hash of the report: 79a5ad352f760e2a829b9ba8a6372b5f271bb7c4793fedbdce94ee50a3a10ab0

Do not edit the generated report. Any changes will be reported as academic dishonesty

## GitHub Info

- GitHub repo name: kobbysika/CS525-p2

- The repository visibility is public.

- The workflow was triggered by kobbysika